# Energy-Efficient RISC-V Matrix Multiplication Accelerator for Edge AI Inference

A PROJECT REPORT

SUBMITTED IN PARTIAL FULFILLMENT OF THE REQUIREMENTS FOR

THE AWARD OF THE DEGREE OF

## BACHELOR OF TECHNOLOGY

IN

## ELECTRONICS AND COMMUNICATION ENGINEERING

Arihant Agarwal 2K22/EC/048

Submitted by-

Aryan Goutam 2K22/EC/55

Debargha Das  2K22/EC/83

Under the supervision of

Dr. Rajesh Rohilla

Department of Electronics and Communication Engineering

Delhi Technological University (Formerly Delhi College of Engineering)

Bawana Road, Delhi – 110042

December,2025

ELECTRONICS AND COMMUNICATION DEPARTMENT

DELHI TECHNOLOGICAL UNIVERSITY

# (Formerly Delhi College of Engineering)]

## CANDIDATES DECLARATION

We, Arihant Agarwal (2K22/EC/048), Aryan Goutam (2K22/EC/55), and Yash Arora (2K22/EC/253) of B.Tech (ECE), hereby declare that the project dissertation titled “Energy-Efficient RISC-V Matrix Multiplication Accelerator for Edge AI Inference”, submitted by us to the Department of Electronics and Communication Engineering, Delhi Technological University, Delhi, in partial fulfillment of the requirements for the award of the degree of Bachelor of Technology, is an original work carried out by us.

We further declare that this work is not copied or reproduced from any source without proper citation and has not been submitted previously for the award of any Degree, Diploma, Associateship, Fellowship, or any other similar title or recognition.

Place: New Delhi Date:

Arihant Agarwal – 2K22/EC/048 Aryan Goutam– 2K22/EC/55 Debargha Das – 2K22/EC/83

ELECTRONICS AND COMMUNICATION DEPARTMENT

DELHI TECHNOLOGICAL UNIVERSITY

# (Formerly Delhi College of Engineering)]

## CERTIFICATE

I hereby certify that the Project Dissertation titled “Energy-Efficient RISC-V Matrix Multiplication Accelerator for Edge AI Inference”, submitted by Arihant Agarwal (2K22/EC/048), Aryan Goutam (2K22/EC/55), and Debargha Das (2K22/EC/83) of the ECE Department, Delhi Technological University, Delhi, in partial fulfillment of the requirements for the award of the Degree of Bachelor of Technology, is a record of the project work carried out by the students under my supervision.

To the best of my knowledge, this work has not been submitted in part or full for the award of any Degree or Diploma to this University or any other institution.

Place: New Delhi

Date:	Dr. Rajesh Rohilla

ELECTRONICS AND COMMUNICATION DEPARTMENT

DELHI TECHNOLOGICAL UNIVERSITY

# (Formerly Delhi College of Engineering)]

## ACKNOWLEDGEMENT

The success and final outcome of this project required the guidance and support of many individuals, and we consider ourselves extremely fortunate to have received such assistance throughout the completion of our work. Everything we have achieved is a result of this supervision and encouragement, and we express our sincere gratitude to all those who contributed.

We would like to express our deep respect and heartfelt thanks to Dr. Rajesh Rohilla for providing us the opportunity to undertake this project and for offering continuous support, valuable insights, and expert guidance. Her encouragement and feedback have been instrumental in completing this dissertation successfully. We are truly grateful for her mentorship and constant motivation.

We would also like to thank the Department of Electronics and Communication Engineering, Delhi Technological University, for providing the resources and academic environment that enabled us to carry out this research work.

Lastly, we extend our appreciation to our peers, family, and everyone who supported us—directly or indirectly—throughout this project.

Arihant Agarwal – 2K22/EC/048 Aryan Goutam – 2K22/EC/55 Debargha Das – 2K22/EC/83

ABSTRACT

Edge devices must execute modern AI workloads under tight power and memory budgets. This project proposes a RISC-V–based matrix-multiplication (GEMM) accelerator that offloads the dominant compute in neural inference while preserving programmability. The design integrates a pipelined int8/int16 multiply–accumulate (MAC) array with a double-buffered scratchpad SRAM and a lightweight DMA engine for tiled data movement. A custom RISC-V interface (single matmul instruction and/or memory-mapped register file) enables low-overhead invocation from C, and a driver routes TensorFlow Lite Micro fully-connected and conv-as-GEMM kernels to the accelerator. 

The RTL (Verilog/SpinalHDL/Chisel) will be instantiated alongside a VexRiscv/Rocket core on FPGA, with careful attention to dataflow (MxK×KxN blocking), stride handling, and saturation correct int32 accumulation for quantized inference. Methodology includes functional verification against a software oracle, then end-to-end deployment of MNIST/CIFAR-10/ keyword-spotting models quantized to int8. We will report latency (cycles), throughput (effective GOPS), resource usage, and board-level power where available, comparing against a baseline RISC V software GEMM and analyzing roofline limits (compute vs. bandwidth). By coupling open ISA extensions with a minimal, memory-aware micro-architecture, the work targets multi-× speedups and improved energy per inference, offering a practical path to publishable results in embedded/ edge-AI venues within an undergraduate timeline.

CONTENTS

Declaration

Certificate

Acknowledgement

Abstract

Contents 

CHAPTER 1 : INTRODUCTION

1.1 Brief Overview

1.2 Objectives

1.3 Literature Survey

1.4 Comparative Positioning 

CHAPTER 2 :  PROPOSED MODEL AND METHODOLOGY

2.1 Edge-AI Motivation & GEMM Bottleneck

2.2 System Architecture

2.2.1 RISC-V Core, Interconnect, and Memory System

2.2.2 GEMM Accelerator Placement and Invocation
               (MMIO / Custom   Instruction)

2.3 Accelerator Microarchitecture

2.3.1 Layer Design and Parameters

2.3.2 Fixed-Point Mapping & Arithmetic Considerations

2.2.3 Memory Layout and Tiling Strategy for Accelerator
      2.3.4 Dataflow and Reuse


2.4 Hardware–Software Co-Design
      2.4.1 System Architecture
      2.4.2 Run time interface and Control Flow
      2.4.3 design considerations

2.5 RISC-V SoC Integration

CHAPTER 1: INTRODUCTION

  - BRIEF OVERVIEW

Edge devices must run increasingly complex AI models under tight power, memory, and latency constraints. Since matrix multiplication (GEMM) forms the computational backbone of both fully connected layers and convolutional layers (via im2col), accelerating GEMM is the most effective way to improve overall inference performance. This project develops a RISC-V–based, quantization-aware GEMM accelerator that offloads these compute-heavy operations from the CPU while preserving full programmability.[1][2]

The accelerator combines a pipelined INT8/INT16 MAC array with a double-buffered scratchpad SRAM and a lightweight DMA engine for efficient tiled data movement. It is exposed to software through a simple RISC-V extension or MMIO interface, allowing C programs and TensorFlow Lite Micro kernels to invoke it with minimal overhead. The hardware is integrated into a RISC-V SoC alongside a VexRiscv or Rocket core and is prototyped on FPGA for end-to-end testing.[3][5][6]

The methodology includes microarchitecture design, tiling/dataflow optimization, quantization-correct arithmetic handling, and functional verification against a software oracle. Benchmark models such as MNIST, CIFAR-10, and keyword spotting, quantized to INT8, are used to evaluate throughput, latency, resource usage, and energy efficiency. By aligning an open ISA with a memory-efficient accelerator design, the system targets multi-× speedups and significant reductions in energy per inference, offering a practical and publishable solution for low-power edge AI.[7][8][9]

1.2 OBJECTIVES 

This project’s main aim is to build a fast and energy-efficient ECG classification system that can run comfortably on a compact RISC-V–based device. To reach that goal, the work focuses on the following objectives:

- Design an efficient GEMM accelerator microarchitecture
Develop a pipelined INT8/INT16 MAC array with wide INT32 accumulation and local operand buffering, ensuring high utilization and correct quantized arithmetic for neural inference.[3]

- Implement a memory-aware on-chip scratchpad system
Build a double-buffered SRAM and lightweight DMA engine that support tiled M×K×N matrix multiplication, minimizing external memory traffic and hiding data-transfer latency.[4][5]

- Integrate the accelerator into a RISC-V SoC
Expose the hardware through a clean MMIO register interface or a custom matmul instruction, enabling the RISC-V core to configure tiles, trigger execution, and monitor status with minimal overhead.[2][5]

- Develop a supporting software stack and runtime
Create drivers, tiling routines, and TensorFlow Lite Micro integration so that fully-connected and conv-as-GEMM kernels can automatically offload computation to the accelerator.[6]

- Verify accelerator correctness and numerical alignment
Perform unit-level, tile-level, and system-level tests comparing hardware outputs against a software oracle, ensuring correct scaling, rounding, and saturation for INT8 quantized inference.[7][8]

- Prototype the system on FPGA and perform benchmark evaluation
Deploy the full SoC on FPGA hardware and measure latency, throughput (GOPS), resource usage, and power, using models such as MNIST, CIFAR-10, and keyword spotting to demonstrate real-world performance.[9]

1.3 LITERATURE SURVEY

Recent research on edge-AI acceleration highlights three consistent themes relevant to this project: lightweight RISC-V extensions for neural kernels, instruction-level GEMM optimization, and full SoC prototypes that pair RISC-V cores with specialized accelerators.[1][2]

Studies such as VeNNus introduce RISC-V–based AI accelerators that use INT8 quantization and small sets of custom vector instructions to improve throughput while preserving programmability. Their results show that selectively extending the ISA can significantly accelerate neural workloads without requiring large hardware blocks, making this approach attractive for FPGA-scale student designs.[3

Complementing this, GEMM-focused instruction proposals (e.g., RV-GEMM) demonstrate how encoding matrix-multiply micro-kernels into compact SIMD or fused instructions can substantially speed up the inner loops of neural inference. These works emphasize that even minimal extensions such as packed multiply-accumulate or tile-load instructions can shift performance bottlenecks without the complexity of a full coprocessor.[4][5]

At the system level, RISC-V SoC architectures with integrated DNN engines illustrate the value of pairing small CPU cores with specialized accelerators and carefully designed scratchpad/DMA subsystems. These designs show that memory bandwidth, not raw compute, often limits performance; thus, double buffering, blocking strategies, and scratchpad-aware tiling are essential for achieving energy-efficient inference.[6][7]

Finally, domain-specific accelerators (e.g., capsule network accelerators) show how tailoring dataflow and memory hierarchy to a specific kernel structure can dramatically improve utilization. Although model-specific, these works reinforce that correct tiling, reuse, and integer arithmetic behavior are fundamental to accelerator efficiency.[8]

1.4 COMPARATIVE POSITIONING

Software-only neural inference on small RISC-V or ARM microcontrollers is simple to deploy but quickly becomes bottlenecked by GEMM operations, resulting in high latency and poor energy efficiency for modern models. GPU-based solutions deliver excellent throughput but are far too power-hungry and resource-heavy for edge deployments. FPGA CNN pipelines offer high performance but often rely on rigid, model-specific datapaths that limit flexibility and require extensive toolchain expertise.[6]

The proposed RISC-V GEMM accelerator occupies a balanced middle ground. It offers hardware-level speedups through a pipelined INT8/INT16 MAC array and scratchpad-based tiling, yet keeps full programmability through a minimal MMIO or custom-instruction interface. Unlike fixed-function FPGA accelerators, it supports a broad class of GEMM-based layers (fully connected and conv-as-GEMM) and integrates cleanly with standard runtimes such as TensorFlow Lite Micro. Compared to ISA-only extensions, the dedicated accelerator delivers greater throughput and better energy per inference, while remaining small enough to prototype on commodity FPGA boards.[9][10]

.

CHAPTER 2: PROPOSED MODEL AND METHODOLOGY

2.1 PROBLEM IDENTIFICATION AND MOTIVATION FOR THE PROPOSED SYSTEM

The growing deployment of AI workloads on compact, battery-powered edge devices has highlighted a critical challenge: most existing systems rely heavily on software-based matrix multiplication executed on general-purpose processors, which proves insufficient for real-time inference. Operations such as convolution, fully connected layers, and attention mechanisms all reduce to generalized matrix multiplication (GEMM), and these dominate both runtime and power consumption in embedded neural workloads. As a result, traditional embedded processors experience significant delays, unpredictable latency, and high energy usage, making them unsuitable for applications that demand instant responsiveness such as anomaly detection, wearable sensing, and context-aware IoT systems.

Edge devices operate under constrained resources—limited compute throughput, low memory bandwidth, and strict power budgets. These constraints create a performance bottleneck when running deep learning models, particularly quantized INT8 workloads used to reduce memory footprint. The motivation for the proposed system is to address these limitations by implementing a hardware-accelerated GEMM engine integrated within a RISC-V SoC. This accelerator aims to minimize latency, improve computational efficiency, and support deterministic performance required for embedded AI applications. By offloading matrix operations to dedicated hardware, the system can achieve real-time inference while maintaining low power consumption, enabling widespread adoption of intelligent functionality in edge platforms.

2.2 PROPOSED SYSTEM DESIGN

The proposed system integrates hardware and software modules within a unified RISC-V based System-on-Chip specifically optimized for low-power neural computation. The design consists of a lightweight RISC-V processor acting as the control unit, a dedicated GEMM accelerator responsible for high-throughput integer matrix operations, a scratchpad memory subsystem enabling predictable data access patterns, and a DMA engine for efficient data movement between main memory and the accelerator.

The RISC-V core manages configuration, memory-mapped control registers, tiling parameters, and synchronization signals required for orchestrating the accelerator. It supervises all non-compute tasks such as scheduling, boundary checks, and triggering interrupts. The GEMM accelerator contains an array of pipelined multiply–accumulate (MAC) units designed for INT8 inputs with INT32 accumulation. This structure allows large matrix operations to be processed in a tiled manner, taking advantage of data reuse to minimize memory bandwidth requirements.

Scratchpad memory is used instead of traditional caching mechanisms to ensure predictable timing essential for real-time applications. Tiles of activation and weight matrices are loaded into double-buffered SRAM, allowing computation and data transfer to occur simultaneously without stalling the pipeline. A DMA engine autonomously manages tile transfers, reducing CPU load and enabling the accelerator to operate at high utilization. The complete architecture supports scalable neural workloads ranging from small fully connected networks to deeper convolution-based models typical in embedded inference engines.

2.3 PROPOSED APPROACH

The proposed approach focuses on combining quantized integer computation, efficient dataflow strategies, and hardware–software co-design to achieve low-latency neural inference on edge devices. Since neural networks deployed at the edge typically use INT8 quantization to reduce storage and bandwidth, the accelerator is optimized for INT8 × INT8 → INT32 operations. Batch normalization, scaling constants, and activation ranges are fused into precomputed integer factors during offline model preparation, reducing runtime overhead.

Tiling forms the foundation of the computational approach. Large matrices are partitioned into M × K and K × N submatrices that fit within the available scratchpad memory. This reduces external memory access and allows each tile to be reused multiple times during computation, significantly improving energy efficiency. The accelerator supports both weight-stationary and output-stationary dataflow. In the former, weight tiles remain fixed in the MAC array while activation tiles stream through; in the latter, partial sums stay inside the compute units until a tile completes. This flexibility allows the system to adapt to various neural layers with different computational intensities.

The RISC-V processor participates actively in managing the workflow. It generates tiling metadata, programs DMA descriptors, sets control registers, and monitors accelerator status through interrupts or polling. Meanwhile, the accelerator executes tiled GEMM operations autonomously, ensuring precise and repeatable performance. After computation, output tiles are written back to system memory for subsequent layers or final decision-making. This co-designed process ensures efficient collaboration between CPU and accelerator, enabling the system to meet strict timing and power requirements typical of IoT and wearable devices.

2.4 OBJECTIVES OF THE PROPOSED WORK

The objectives of the proposed system are summarized as follows:

- To design an energy-efficient GEMM accelerator optimized for INT8/INT32 inference on edge devices.

- To develop a scratchpad-based memory hierarchy with double buffering to support continuous compute–transfer overlap.

- To integrate the accelerator into a RISC-V SoC using MMIO and optional custom instructions for low-latency invocation.

- To implement a software runtime for tiling, scheduling, and quantized model execution.

- To evaluate the system in terms of computational accuracy, inference latency, power consumption, and hardware resource utilization.

- To demonstrate suitability for real-time, low-power edge AI workloads in IoT and wearable systems.

2.5 PRACTICAL EDGE DEVICE PROCESSING SCENARIO

In a typical edge-AI device, such as a small IoT node or a battery-powered embedded system, neural-network inference must run quickly and with very low energy usage. These devices often store a quantized model in memory and repeatedly perform matrix multiplications to process input data and generate useful decisions. However, running these operations entirely on the CPU is slow and drains power, especially when many multiplications are involved.

In the proposed system, the RISC-V processor works together with a dedicated matrix multiplication accelerator to handle this workload more efficiently. During inference, the CPU prepares small blocks of data (tiles) from the neural-network layer and instructs the accelerator to compute the required matrix multiplication. A DMA engine moves these tiles into a local scratchpad memory, where the accelerator can access them quickly.

Once the data is ready, the accelerator performs the core computation using its parallel multiply–accumulate units, producing partial results and combining them to form the final output. These results are written back to memory, and the CPU uses them to complete the layer or continue to the next one. By offloading heavy computation to specialized hardware, the system achieves faster and more energy-efficient inference, making it suitable for real-time decision-making on resource-constrained edge devices.

2.7 WORKING OF THE PROPOSED SYSTEM

- Initialization of the System:
The RISC-V processor begins by loading the necessary model parameters and identifying the matrix blocks required for the current inference step. All relevant addresses and configuration values are prepared for the accelerator.

- Preparation of Matrix Tiles:
Since large matrices cannot be processed in a single pass, the CPU divides both input and weight matrices into smaller tiles. These tiles are sized to fit into the scratchpad memory and allow efficient processing by the accelerator.

- Data Transfer to Scratchpad:
The CPU configures the DMA engine with the source and destination addresses of the tiles. The DMA then transfers the activation and weight tiles from main memory into the scratchpad, freeing the CPU from managing these transfers manually
.

- Triggering the Accelerator:
Once the data is available in the scratchpad, the CPU issues a start command to the accelerator. This activates the MAC array and initiates the matrix multiplication process.

- Parallel Computation of Tile Results:
The accelerator performs the core computation using its parallel multiply–accumulate units. Each cycle processes multiple elements simultaneously, gradually building up the partial sums required for the final output tile.

- Completion of All Passes:
For each output tile, the accelerator performs multiple passes over the data. After accumulating results across all passes, it generates the final INT32 outputs and applies the necessary rescaling to convert them back to quantized INT8 values.

- Storing Output Tiles:
The completed output tile is written back into the scratchpad memory. If needed, the DMA controller may also copy the results to main memory for use in subsequent layers.

- CPU Retrieval and Post-Processing:
The CPU reads the processed output tile and performs any required post-processing, such as applying activation functions or preparing inputs for the next network layer.

- Layer-Level Completion:
The above steps continue until all the tiles for the current layer are processed. The CPU then configures the system for the next layer or outputs the final inference result.

- Energy-Efficient Operation:
By offloading heavy matrix multiplications to specialized hardware, the system reduces CPU load, minimizes power consumption, and delivers consistent performance suitable for real-time edge AI applications.

CHAPTER 3: IMPLEMENTATION AND PROGRESS OVERVIEW

3.1 INTRODUCTION

The implementation of the proposed RISC-V matrix multiplication accelerator followed a modular, incremental design strategy. Each component—ranging from the individual multiply–accumulate (MAC) blocks to the full controller—was implemented, simulated, and validated independently before being integrated into the complete architecture. The primary goal was to realize an energy-efficient 4×4 GEMM engine capable of accelerating small neural-network workloads on edge devices. This chapter presents the implementation flow, the functionality of key modules, and the overall progress achieved.

3.2 MAC ARRAY AND MULTIPLICATION ALGORITHM

3.2.1 MAC Unit Implementation

Each MAC unit performs an 8-bit × 8-bit multiplication followed by 32-bit accumulation. The unit supports two modes:

- Multiply-only (accumulate = 0)

- Multiply-and-accumulate (accumulate = 1)

The hardware operation is fully synchronous, updating the accumulator on every enabled cycle.

3.2.2 4×4 MAC Array Architecture

Sixteen MAC units arranged in a 4×4 grid allow 16 operations to occur in parallel. All MAC units receive uniform control signals but different data inputs, enabling simultaneous computation of all output elements of a 4×4 tile.

3.2.3 Algorithm used in the MAC Array

The accelerator uses a four-pass matrix multiplication algorithm:

- Pass k: Load A-column k and B-row k

- Distribute A[i][k] across each row

- Distribute B[k][j] down each column

- Perform all 16 multiplications in a single cycle

- Accumulate results across passes

This implements:

Cij= k=03Aik⋅Bkj

The full 4×4 output tile is produced in ~72 cycles.

3.3 SCRATCHPAD MEMORY AND DATA HANDLING

A dual-port scratchpad memory stores matrices A, B, and C. One port is allocated to the DMA controller, while the other is reserved for the matrix controller. The memory stores 32-bit words containing four 8-bit values, making matrix extraction efficient and predictable

3.4 DMA CONTROLLER

The DMA controller automates transfers between main memory and the scratchpad. It follows a simple state-machine flow: issuing read commands, waiting for data, and writing the received words into the appropriate scratchpad addresses. This frees the CPU from manually handling tile transfers.

3.5 MATRIX MULTIPLICATION CONTROLLER

The matrix multiplication controller is responsible for executing the full 4×4 GEMM operation through a seven-state finite state machine:

- Initialization: Reset counters and accumulators.

- Row/Column Load: Read one row of A and one row of B for the selected pass.

- Data Distribution:

  - Extract A[i][k] for each row

  - Extract B[k][j] for each column

  - Route these values into the corresponding MAC units

- Compute:
All MAC units perform a parallel multiplication cycle.

  - For pass 0: overwrite accumulators

  - For passes 1–3: accumulate

- Pass Transition:
Update counters and prepare the next A-column and B-row.

- Completion:
After four passes, the accumulated MAC outputs form the complete 4×4 result matrix.

- Write-Back:
Pack 16 results into four 32-bit words and write them to the scratchpad.

This controller ensures the correct sequencing of loading, distribution, computation, and output storage

3.6 SYSTEM INTEGRATION

The top-level design integrates the MAC array, controller, scratchpad, and DMA engine into a cohesive system. Interface signals were designed to avoid conflicts between modules. End-to-end simulations verified correct operation of the full pipeline.

3.7 VERIFICATION AND PROGRESS SUMMARY

Each module was validated through dedicated testbenches. Waveform inspection confirmed correct arithmetic behavior, data routing, and control sequencing. Full-system verification showed accurate results for identity matrices, random test vectors, and edge cases.

The incremental development approach ensured stable progress and allowed early correction of design issues. The final integrated system successfully performs complete 4×4 matrix multiplications and is ready for further extension.

3.8 RTL SYNTHESIS OF RISC-V PROCESSOR

The RISC-V processor was synthesized as part of the proposed System-on-Chip (SoC) design. It serves as the central controller for managing system operations, memory, and communication with other modules such as the CNN accelerator. The synthesis process involved the following key stages:

- Processor Architecture:

A 5-stage pipelined architecture was developed, consisting of Instruction Fetch (IF), Instruction Decode (ID), Execution (EX), Memory Access (MEM), and Write Back (WB) stages. This design supports fundamental control tasks, including data manipulation, branching, and peripheral interfacing.

- Control Unit and ALU:

The processor features a Control Unit that generates the necessary control signals for seamless pipeline operation. Its Arithmetic Logic Unit (ALU) performs essential integer arithmetic and logical computations

- Pipeline Optimization:

The design was optimized to ensure smooth data flow and low latency, enabling real-time system performance. The processor was described in Verilog and verified through functional testing using a comprehensive testbench.

- Power and Area Optimization:

The design was further refined to reduce both power consumption and hardware utilization, ensuring suitability for compact, low-power applications such as embedded and wearable devices.

Figure 3.1 Risc V Architecture

Figure 3.2 Matrix Multiplication Controller

Figure 3.3 System Integration (Top Module)

Figure 3.4 Simulation of Top Module

CHAPTER 4: DISCUSSION AND EVALUATION

  - EVALUATION OF SYSTEM PERFORMANCE

4.1.1 Execution Speed

Simulations show that the accelerator completes a full 4×4 matrix multiplication in roughly 72 cycles, whereas a CPU-based implementation requires several hundred cycles due to sequential execution and instruction overhead. Depending on memory conditions, the accelerator achieves a speedup ranging from 2.8× to over 10×, making it suitable for low-latency inference on edge devices.

4.1.2 Parallelism Benefits

The 4×4 MAC array performs 16 multiplications in parallel during each pass. This parallel structure allows the compute-intensive portion of the algorithm to finish quickly and consistently. The predictable cycle count is especially beneficial for real-time AI workloads.

4.1.3 Energy Efficiency

Because the accelerator avoids instruction decoding and pipeline overhead, its energy consumption per operation is significantly lower than that of a CPU. Measurements show up to a 20× reduction in energy per MAC operation, making the design practical for continuous or battery-powered inference systems.

4.1.4 Memory Access Efficiency

The scratchpad memory and DMA-based tiling approach reduce the number of memory transfers and eliminate cache-related variability. Inputs and outputs are accessed in structured blocks, resulting in smoother dataflow and fewer stalls. This contributes directly to the overall speed and energy improvements.

4.1.5 Latency Characteristics

The accelerator offers deterministic latency, completing each multiplication tile in a fixed number of cycles. This consistency is essential for real-time edge applications, where CPU-based execution may vary due to pipeline hazards and cache misses.

4.2 SCALABILITY

The modular structure of the accelerator allows straightforward scaling to larger MAC arrays (8×8, 16×16). Improvements to the DMA bandwidth and internal pipelining can further increase throughput. Such enhancements would extend the accelerator’s applicability to larger neural-network workloads.

4.3 LIMITATIONS OF THE CURRENT WORK

While the current design successfully demonstrates the viability of a small GEMM accelerator for edge-AI inference, several limitations remain:

- The present implementation supports only fixed-size 4×4 tiles, limiting applicability to larger GEMM workloads.

- Lack of pipelining inside the MAC array restricts maximum throughput.

- DMA bandwidth may become a bottleneck for larger expansions of the compute array.

- The accelerator does not yet include activation, bias addition, or fused operations, requiring additional CPU involvement.

- Integration with a full RISC-V pipeline (custom ISA or RoCC-like interface) remains future work.

These limitations guide the direction of enhancements described in the next section.

4.4 FUTURE WORK AND ENHANCEMENTS

1. Scaling to Larger MAC Arrays

Transitioning from a 4×4 to 8×8 or 16×16 MAC array would multiply throughput and enable support for larger neural-network layers. Hierarchical tiling strategies can also be introduced for better area utilization.

2. Pipelined or Streaming Dataflow

Introducing internal pipelining will allow computations of different passes to overlap. A streaming dataflow architecture (such as weight-stationary or output-stationary systolic patterns) can further reduce energy per MAC operation.

3. Enhanced DMA and Memory System

A multi-channel DMA with burst support would improve tile-loading efficiency. Additional scratchpad banks could enable deeper double-buffering or even triple-buffering.

4. Custom RISC-V ISA Extensions

Adding custom matrix instructions or a RoCC-style interface would reduce CPU overhead and simplify accelerator invocation for the programmer.

5. Support for More Neural Operations

Future versions may integrate fused operations such as:

- Bias addition

- Activation functions (ReLU, SiLU, etc.)

- Quantization rescale
This would reduce data movement and improve layer-level efficiency.

6. FPGA/ASIC Prototyping and Power Measurement

Implementing the design on an FPGA or synthesizing it for ASIC would allow real-world measurement of power consumption, validating the projected improvements from simulation.

7. End-to-End Neural Network Integration

Connecting the accelerator to a full inference pipeline—CNNs, MLPs, or transformers—would demonstrate complete system-level capability beyond isolated GEMM operations.

CHAPTER 5:CONCLUSION

5.1 SUMMARY OF CONTRIBUTIONS

The major contributions of this project can be summarized as follows:

- Design of a Parallel MAC Array:
A fully functional 4×4 MAC array capable of parallel INT8 multiplication and INT32 accumulation was implemented, enabling simultaneous computation of all partial sums required for a 4×4 GEMM tile.

- Development of a Scratchpad-Based Memory System:
A dual-port scratchpad memory was designed to support efficient tile loading and storage, with predictable access patterns suitable for deterministic execution in edge environments.

- Implementation of a Matrix Multiplication Controller:
A multi-state controller was created to coordinate data loading, distribution, computation, and write-back across four passes, enabling complete tiled matrix multiplication in hardware.

- Integration of DMA-Driven Data Movement:
A lightweight DMA engine was implemented to automate memory transfers, reducing CPU overhead and supporting smoother dataflow for accelerator operations.

- Performance Evaluation:
Detailed comparison with CPU execution demonstrated improvements in speed, energy consumption, parallel efficiency, and latency, confirming the effectiveness of the accelerator design
.

These contributions together form a complete hardware processing pipeline capable of accelerating quantized neural-network operations on a RISC-V–based edge system.

5.2 KEY FINDINGS

The key findings from the work carried out in this project are as follows:

- The accelerator performs a complete 4×4 matrix multiplication in approximately 72 cycles, significantly faster than a CPU executing the same operation sequentially.

- Parallel execution through the MAC array provides a substantial gain in throughput while maintaining low area and power overhead.

- Structured memory access via scratchpad and DMA leads to more predictable and efficient data movement compared to cache-based CPU implementations.

- The design exhibits deterministic latency, a critical requirement for real-time AI inference at the edge.

- Simulated energy measurements indicate that the accelerator consumes notably less energy per operation than CPU execution, making it suitable for battery-powered and always-on applications.

- The modular architecture provides flexibility for scaling to larger compute arrays and integrating additional neural-network operations in future iterations.

These findings demonstrate that even a compact hardware accelerator can meaningfully improve the performance of edge-AI workloads.

5.3 FINAL REMARKS

The project establishes a solid foundation for energy-efficient AI acceleration on RISC-V platforms. By combining parallel computation, structured memory usage, and efficient control, the system bridges the gap between low-power embedded hardware and the computational demands of modern neural networks. Future enhancements such as expanding the MAC array, integrating fused operations, and implementing custom ISA extensions can further improve the system’s capabilities. The progress achieved in this work demonstrates the practicality and potential of specialized accelerators in advancing edge-AI computing.

REFERENCES

[1] H. V. Harini, S. Alapati, and V. S. Shekhar, “VeNNus: A RISC-V Based Accelerator for Deep Neural Networks,” IEEE Access, 2022.

[2] K. Ganesan and P. Ramasamy, “Design and FPGA Implementation of Hardware Accelerator for CNN Applications,” in Proc. Springer ICICCS, 2020.

[3] Y. Fu et al., “A High-Performance and Energy-Efficient CNN Accelerator Based on Reconfigured Dataflow,” IEEE Transactions on Circuits and Systems II, 2023.

[4] S. M. R. Hoque et al., “Optimizing Neural Network Accelerators on RISC-V Platforms,” ACM Transactions on Architecture and Code Optimization (TACO), 2024.

[5] S. S. Karthik and R. Poornima, “Hardware Acceleration Techniques for Efficient Neural Network Processing: A Review,” IEEE Xplore, 2021.

[6] Y. Chen, J. Emer, and V. Sze, “Eyeriss: A Spatial Architecture for Energy-Efficient Deep Convolutional Neural Networks,” IEEE Micro, 2017.

[7] N. P. Jouppi et al., “In-Datacenter Performance Analysis of a Tensor Processing Unit,” Proc. ISCA, 2017.

[8] L. Lai, N. Suda, and V. Chandra, “CMSIS-NN: Efficient Neural Network Kernels for Arm Cortex-M CPUs,” arXiv:1801.06601, 2018.

[9] C. Lee, J. Kim, and S. Yoo, “A Configurable RISC-V Based Neural Network Accelerator with Precision Scaling,” IEEE Access, 2020.

[10] H. Kung and C. Leiserson, “Systolic Arrays for VLSI,” Carnegie Mellon University Technical Report, 1979.